dba IMV NEVADA

MATERIAL SAFETY DATA SHEET

Identity:

Trowel-Mate™

Section I

Manufacturer’s Name                                  Emergency Telephone Number

IMV Nevada                                                   800-243-0513

775-372-5341

Address:					  Telephone Number for Information

HC 70 Box 549                                                775-372-5341

498 E. Imvite Rd.

Amargosa Valley, Nevada  89020         Date Prepared:  March 5, 2015

SECTION II –HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

Hazardous Components: Respirable dust may contain crystalline silica in the form of quartz

| Ingredient | Chemical Formula | Common Name | Concentration, % | CAS |
| --- | --- | --- | --- | --- |
| Saponite | Si7.34(Al0.66Mg6 O20 ((OH)4 | Saponite | 1-99% | 1319-41-1 |
| Bentonite | Si8(Al3.34Mg0.66 O20 ((OH)4 | Bentonite | 1-99% | 1302-78-9 |
| Crystalline silica | SiO2 | Quartz | < 1% | 14808-60-7 |

(Crystalline silica is reported as total silica and not just the respirable fraction.)

Trade names covered by this MSDS are listed in Section VIII

SECTION III –PHYSICAL/CHEMICAL CHARACTERISTICS

Boiling Point:	N/A     Specific Gravity	(H2O=1)   1.01-2.4

Vapor Pressure (mm Hg.):   None	    Melting Point:              None

Vapor Density (AIR=1)        None          Evaporation Rate:       None

Solubility in Water:        Insoluble in water

Appearance and Odor: White, light gray, tan or reddish tan, granular or powder. Earthy odor when wet.

SECTION IV –FIRE AND EXPLOSION HAZARD DATA

Flash Point (method used): non-flammable

Flammable Limits:  None	LEL: None			UEL: None

Extinguishing Media:   None Required

Special Fire Fighting Procedures:  None

Unusual Fire and Explosion Hazards: None

SECTION V -- REACTIVITY DATA

Stability: Unstable (  )   Stable   ( X )				Conditions to Avoid:  None

Incompatibility (Materials to Avoid):  None

Hazardous Decomposition or Byproducts:  None

Hazardous Polymerization:  Will Not Occur

SECTION VI – HEALTH HAZARD DATA

Route(s) of Entry:  Inhalation?  Yes  Skin?  No   Ingestion?  No

| Ingredient | OSHA PEL, TWA 8/40h (mg/m3) | ACGIH TLV, TWA 8/40h (mg/m3) | NIOSH REL, TWA 8/40h (mg/m3) | NIOSH IDLH (mg/m3) |
| --- | --- | --- | --- | --- |
| Saponite | 10 | 10 | n/a | n/a |
| Bentonite |  |  |  |  |
| Crystalline Silica, SiO2 | 10/(SiO2% + 2) (respirable) | 0.025 (respirable) | 0.05 (respirable) | 50 |

Health Hazards (Acute and Chronic)

Smectite is not listed by MSHA, OSHA or IARC as a carcinogen.  However, this product may contain trace amounts of crystalline silica in the form of quartz.  Quartz has been classified by IARC as a Group 1 carcinogen to humans when inhaled.  Inhalation of silica can also cause a chronic lung disorder (silicosis).

Emergency and First Aid Procedures:  For dust in eyes, wash immediately with water.  If irritation persists, seek medical attention.  For gross inhalation, remove person immediately to fresh air and give artificial respiration as needed, seek medical attention as needed.

SECTION VII – PRECAUTIONS FOR SAFE HANDLING AND USE

Steps to be taken in Case Material is Released or Spilled:

Spills: Use dustless methods (vacuum) and place into closable container for disposal, or flush with water.  Do not dry sweep.  Care should be taken to avoid high aerosol dust concentrations. Wear protective equipment specified below.  Spilled material may become very slippery when wet with water, grease, oil, gasoline, or solvents.

Waste Disposal Method: Dispose in accordance with Federal, State and Local regulations.

Precautions to Be Taken in Handling and Storing:  Avoid breakage of bagged material or spills of bulk material.  See control measures in Section VIII.

Other Precautions:  Use dustless systems for handling, storage, and clean up so that airborne dust does not exceed the PEL.  Use adequate ventilation and dust collection.  Practice good housekeeping.  Do not permit dust to collect on walls, floors, sills, ledges, machinery, or equipment.  Maintain, clean, and fit test respirators in accordance with OSHA regulations.  Maintain and test ventilation and dust collection equipment.  Wash or vacuum clothing which has become dusty.  See also control measures in Section VIII.

See OSHA Hazard Communication Rule 29 CFR Sections 1910.1200, 1915.99, 1917.28, 1918.90, 1926.59 and 1928.21 and state and local worker or community “right to know” laws and regulations.  We recommend that smoking be prohibited in all areas where respirators must be used.  WARN YOUR EMPLOYEES (AND YOUR CUSTOMERS-USERS IN CASE OF RESALE) BY POSTING AND OTHER MEANS OF THE HAZARDS AND OSHA PRECAUTIONS TO BE USED.  PROVIDE TRAINING FOR YOUR EMPLOYEES ABOUT THE OSHA PRECAUTIONS.

See also American Society for Testing and Materials (ASTM) “Standard Practice for Health Requirements Relating to Occupational Exposure to Quartz Dust.

SECTION VIII – CONTROL MEASURES

Ventilation:

Local Exhaust: Use sufficient local exhaust to reduce the level of dust

to the PEL.  See ACGGIH “Industrial Ventilation, A manual of

Recommended Practice,” the latest edition.

Mechanical:  See “Other Precautions” under Section VII

Special:  See “Other Precautions” under Section VII

Other:  See “Other Precautions” under Section VII

Respiratory Protection: Use appropriate NIOSH/ OSHA/ MSHA approved respirators if airborne concentrations exceed PELs.

Eye Protection:  Wear protective shield (safety glasses) when exposed to dust particles

Protective Gloves: Optional

Other Protective Clothing & Equipment:  Optional

Work/Hygienic Practices:  Avoid creating and breathing dust.  See “Other Precautions” under Section VII

The information and recommendations contained herein are based upon data believed to be correct.  However, no guarantee or warranty of any kind, express or implied, is made with respect to the information contained herein.  We accept no responsibility and disclaim all liability for any harmful health effects which may be caused by purchase, resale, and use of exposure to our smectite clay.  Customers-users or Smectite clay must comply with all applicable health and safety laws, regulations and orders.

TRADE NAMES: